Network Equipment Communication Protocol Message Specification V1.0

Revision History

| Version | Creation/Revision Date | Author/Modifier | Revision Description |
| --- | --- | --- | --- |
| 1.0 | 2017-03-28 | dylan | Added |
| 1.1 | 2017-04-07 | dylan | Corrected some errors |
| 1.2 | 2017-04-10 | Dylan | Added the TFLV structure description |
| 1.3 | 2017-06-29 | Dylan | Added some parameters |
| 1.4 | 2017-11-20 | Dylan | Added IMEI number support and the Do Not Disturb function |
| 1.5 | 2018-01-05 | Dylan | Added registration parameter - Parameter Synchronization Identifier Added control parameter - Restore Factory Settings |
| 1.6 | 2018-05-03 | Liya | Added configuration parameter - Early Medication Retrieval Validity Period Added configuration parameter - Alarm repeat function setting Added control parameter - Early Medication Retrieval |
| 1.7 | 2018-09-20 | Liya | Added Event Parameters-Pill Tray Reset Report |
| 1.8 | 2018-11-13 | Liya | Added IoT SIM card CCID reporting Added network configuration result reporting |
| 1.9 | 2018-12-12 | dylan | Added User Binding Relationship(0x800A) Added Factory Reset Report(0xC501) |
| 1.10 | 2018-12-18 | dylan | Added Device Startup Report(0xC206) Added Device Startup Report(0xC601) |
| 1.11 | 2019-01-14 | Dylan | Added Medication Retrieval Trigger Identifier(0xC207) |
| 1.12 | 2019-01-17 | Dylan | Added Medication Retrieval Time(0xC208) |
| 1.13 | 2019-03-22 | dylan | Added Online Hold Time(0x800B) Added Set Heartbeat Interval(0xA011) Added Set Online Hold Time(0xA012) |
| 1.14 | 2019-06-20 | Dylan | The definitions of “Byte Array” and “String” were exchanged; existing parameters similar to byte arrays were changed to the String type. |
| 1.15 | 2019-12-06 | Dylan | Protocol optimization |
| 1.16 | 2020-01-15 | Dylan | Alarm duration (0x1011) unit changed from seconds to minutes; Added parameter - Additional Function Identifier (0x8081) Added parameter - Custom Number (0x8082) Removed the reserved parameter - Set Online Hold Time (0xA012) Added parameter - Set Server IP (0xA021) Added parameter - Set Server Domain Name (0xA022) Added parameter - Set Server Port (0xA023) The value definition for Early Medication Retrieval (0xA123) was removed Added parameter - Device Re-registration (0xA004) |
| 1.17 | 2020-06-16 | Dylan | Added alarm parameters: 0x1027, 0x1028, 0x1029, 0x1037, 0x1038, and 0x1039 |
| 1.18 | 2022-05-13 | Dylan | Added parameter - Medication Status (0x8101) Added parameter - DC Power Status (0x8109) Added parameter - Pill Tray Reset Fault (0x8122) Added parameter - Emergency Call Status (0x8123) Added parameters - Alarm Status 1-9 (0x8131-0x8139) Added parameter - Emergency Call Trigger Time (0x1061) Added parameter - Key Tone Switch (0x1062) Added parameter - Alarm Ringtone Type (0x1012) Removed parameter - Alarm Ringtone Hold Duration (0x1011) |
| 1.19 | 2022-07-25 | Dylan | Added parameter value - Battery Status (0x8104): Value 4 - No battery |
| 1.20 | 2022-08-29 | Dylan | Reorganized parameter tables |
| 1.21 | 2022-09-05 | Dylan | Added all M2 parameters (device type 0x02) |
| 1.22 | 2022-09-08 | Dylan | Added upgrade-related packet types (0x0E, 0x8E, 0x0F, 0x8F) |
| 1.23 | 2022-09-27 | Dylan | Added a parameter for the remote upgrade function (upgrade timeout) |
| 1.24 | 2022-10-28 | Dylan | Changed the M2 Battery Status values (0x8104) |
| 1.25 | 2024-05-06 | ZhouZS | Corrected an error in the horizontal message-structure illustration |
| 1.26 | 2025-12-19 | ZhouZS | Added parameter - Alarm Ring Pause Duration (0x1063) Added parameter - Rotate to Specified Medication Compartment (0xA124) Added parameter - Medication Pause Duration Setting (0xA125) Added parameter - Pill Tray Rotation Status (0x8140) Added parameter - Medication Pause Status (0x8141) Adjusted parameter for M1/M2 consistency - Alarm Ringtone Type (0x1012) Adjusted parameter for M1/M2 consistency - Volume Level (0x1013) Adjusted parameter for M1/M2 consistency - Medication Retrieval Warning Time (0x1017) Adjusted parameter for M1/M2 consistency - Missed Medication Retrieval Time (0x1018) Adjusted parameter for M1/M2 consistency - Current Medication Compartment Number (0x101A) Adjusted parameter for M1/M2 consistency - Battery Level (0x8103) Adjusted parameter for M1/M2 consistency - Battery Status (0x8104) |

Terminology

| Proper Noun | Definition |
| --- | --- |
| Byte Sequence | Refers to the storage order of bytes when multi-byte data is stored in computer memory or transmitted over the network. |
| Host Byte Order | The low address stores the low byte of word data, and the high address stores the high byte of word data. |
| Network Byte Order | The high byte of word data is stored in the low address, while the low byte of word data is stored in the high address |

Contents

1. Message Specification	8

1.1. Data Type Definition	8

1.1.1. Data Type Definition	8

1.1.2. Data Processing Rules	8

1.1.3. Byte Order	9

2. Message Format Description	9

2.1. Message Structure Description	9

2.2. TFLV Structure Description	10

3. Message Transmission Protocol	12

4. Message Application Processing Modes	13

4.1. Mode 1: Synchronous No-Response Processing Mode	13

4.2. Mode 2: Synchronous Response Processing Mode	13

4.2.1. One request requires one request packet, and one request packet requires one response packet	13

4.2.2. One request requires one request packet, and one request packet requires n response packets	14

4.2.3. One request requires n request packets, and one request packet requires one response packet	15

5. Exception Handling	16

5.1. Response Timeout Exception	16

5.2. Abnormal Packet Status Identification	16

5.3. Unsupported Protocol Version	16

5.4. Unsupported Device Type	16

5.5. Unsupported Packet Type	16

5.6. Inconsistent Device ID	16

5.7. CRC Check Exception	17

6. Packet Type Description	18

7. Interaction Modes	19

7.1. Interaction Description	19

7.1.1. Registration Phase	19

7.1.2. Running Phase	20

8. Appendix	32

8.1. Code Table	32

8.1.1. Status Identifier	32

8.1.2. Function Identifier	32

8.1.3. Device Type	33

8.1.4. Device ID	33

8.1.5. TFLV Structure - Status Identification	33

8.1.6. TFLV Structure - Parameter Type	34

8.2. CRC Algorithm C Code	34

8.3. TAG Definition - Device Type 01	37

8.4. TAG Definition - Device Type 02	47

- Message Specification

  - Data Type Definition

    - Data Type Definition

| Data Type | Definition |
| --- | --- |
| INT8U | Unsigned integer (1 byte) |
| INT8S | Signed 8-bit integer (1 byte) |
| INT16U | Unsigned integer (2 bytes) |
| INT16S | Signed integer(2 bytes) |
| INT32U | Unsigned integer(4 bytes) |
| INT32S | Signed integer(4 bytes) |
| INT64U | Unsigned integer(8 bytes) |
| INT64S | Signed integer(8 bytes) |
| FLOAT | Single-precision floating-point number |
| DOUBLE | Double-precision floating-point number |
| STRING | Character string |
| ARRAY | Byte array |

    - Data Processing Rules

- The content to be filled in for each field is described in the sections for the corresponding packet types.

- When “Length” is n, the length of this field is fixed at n bytes.

When “Length” is [m,n], the minimum length of this field is m bytes and the maximum length is n bytes.

- When “Mandatory/Optional” is M, this element is mandatory and must be present.

When “Mandatory/Optional” is O, this element is optional and should be filled in as required.

    - Byte Order

The byte order for data transmission in this protocol is host byte order.

- Message Format Description

  - Message Structure Description

The basic message structure is shown in the table below:

| Header | Length | Status | Version | Serial Number | Subserial number | Subpacket amount | Flag | Device Type | Device number | Packet Type | Application Data | Check code |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

The structure fields are described in the table below:

| Name | Instruction | Data Type | Length | Mandatory/Optional |
| --- | --- | --- | --- | --- |
| Header | Fixed to 0xAA | INT8U | 1 | M |
| Length | Length of total bytes amount (From “Status” to “Application Data”) | INT16U | 2 | M |
| Status | Status of the data packet，value refers to"Status Identification" | INT8U | 1 | M |
| Version | Protocol version, fixed to 0x01 | INT8U | 1 | M |
| Serial number | Packet sequence number, incremented each time a new packet is sent，0~65535 | INT16U | 2 | M |
| Subserial number | A large data packet is split into several sub-packets and sent. Identifies the sequence number of the subpacket. The sequence number starts from 0. | INT8U | 1 | M |
| Subpacket total amount | A large data packet is split into several sub-packets and sent. Identifies the total number of subpackets | INT8U | 1 | M |
| Flag | Various functional parameters that identify the data packet. For values, see"Functional Identification" | INT8U | 1 | M |
| Device type | Device type, see value Device type | INT8U | 1 | M |
| Device number | Unique identifiers for different devices. For values, see"device number” | INT64U | 8 | M |
| Packet type | Packet type, please refer to "《Packet type description》" for the value. | INT8U | 1 | M |
| Application Data | Indicates the application data, see the description of each packet type for the specific format. | INT8U | [0,1400] | O |
| Check code | Indicates that the data from the "Length" to the end of the "Application data" is data-checked, and the checking algorithm is CRC16; for the implementation of Algorithm C, see "CRC16 Algorithm". | INT16U | 2 | M |

  - TFLV Structure Description

Purpose: used to convey the value of an element in the protocol.

The basic structure is shown in the table below:

Tag

Flag

Length

Data Value

Status

Type

Figure 2.2-1

The structure fields are described in the table below:

| Field Name | Field Description | Field Type (see 1.1.1 Data Type Definition") | Length | Mandatory/Optional |
| --- | --- | --- | --- | --- |
| Tag | The number of the information carried by this TFLV. | INT16U | 2 | M |
| Flag | Bits 0~4 indicate the data type. For values, see"Appendix - Code List - TFLV Structure - Parameter Type". Bits 5~7 represent status, see the value"Appendix - Code List - TFLV Structure - Status Identification" | INT8U | 1 | M |
| Length | Length of data value | INT8U | 1 | M |
| Data Value | Data information | ARRAY | [0,255] | O |

- Message Transmission Protocol

TCP, UDP, and HTTP are supported, with TCP preferred.

When using HTTP, the content-type is application/octet-stream.

- Message Application Processing Modes

  - Mode 1: Synchronous No-Response Processing Mode

Figure 4.1-1

This mode is used when Terminal A actively sends data to the device and does not require a response from the device.

  - Mode 2: Synchronous Response Processing Mode

Figure 4.2-2

This mode is used when Terminal A sends data to the device and requires Terminal B to respond. If Terminal B does not respond within the specified time, Terminal A retransmits the packet. If the device still does not respond after the specified number of retransmissions, communication between Terminals A and B is considered abnormal and the connection should be disconnected.

In this mode, the data protocol requires each packet to stay within the maximum data length specified by the application protocol. When the total length of a request or response exceeds that maximum, the data must be split into subpackets. Therefore, three cases must be considered:

    - One request requires one request packet, and one request packet requires one response packet

Figure 4.2-2

After one interaction is completed, refer to the description of “Mode 2: Synchronous Response Processing Mode”.

In this mode, the subpacket information fields (subpacket sequence number and total number of subpackets) are both set to 0.

    - One request requires one request packet, and one request packet requires n response packets

Figure 4.2-4

Terminal A sends a request and receives a response from Terminal B. If parsing the response shows that the response data requires multiple response packets, the subpacket fields in the previous request packet must be updated to identify the response sequence requested, and the packet must be sent to Terminal B again.

Requester

When sending a request packet, set both subpacket information fields (subpacket sequence number and total number of subpackets) to 0.

When a response packet is received and the total number of subpackets is not 0, the response requires n response packets. Increment the subpacket sequence number in the original request packet by 1, set the request packet total number of subpackets to the value in the response packet, and continue sending request packets. When the subpacket sequence number plus 1 equals the total number of subpackets, the response has been fully received.

Sender

When a request packet is received with both the total number of subpackets and the subpacket sequence number set to 0, determine whether the response requires multiple response packets. If so, send the first packet and set the subpacket data in the request packet to the total number of response packets, informing the requester of the total number of subpackets to be returned.

When the total number of subpackets is not 0, return the response subpacket corresponding to the subpacket sequence number in the request packet.

    - One request requires n request packets, and one request packet requires one response packet

Figure 4.2-4

Because one request must be divided into multiple request packets, each request packet is processed as an independent request and the device must return the corresponding response information.

See “Mode 2: Synchronous Response Processing Mode - One request requires one request packet, and one request packet requires one response packet”.

- Exception Handling

  - Response Timeout Exception

When the sending terminal does not receive a response from the receiving terminal within the specified time, it retransmits the data. If no response is received after the specified number of retransmissions, communication between the two terminals is considered abnormal and the connection should be disconnected.

  - Abnormal Packet Status Identification

If the sending terminal receives a non-zero status identifier, the previously sent packet could not be recognized by the receiving terminal. A notification should be provided.

If the message header received by the receiving terminal is abnormal, set the status identifier of the original received packet to the corresponding error identifier and return it to the sending terminal.

  - Unsupported Protocol Version

If a terminal receives a packet with an unsupported protocol version, it must set the packet status bit to the corresponding status and return the adjusted original packet to the sender.

  - Unsupported Device Type

If a request packet contains an unsupported device type, set the packet status bit to the corresponding status and return the adjusted original packet to the sender.

  - Unsupported Packet Type

If a request packet contains an unsupported packet type, set the packet status bit to the corresponding status and return the adjusted original packet to the sender.

  - Inconsistent Device ID

When a terminal receives a packet, it checks whether the device IDs match. If they do not match, set the packet status bit to the corresponding status and return the adjusted original packet to the sender.

  - CRC Check Exception

If a CRC error occurs in a received packet, discard the packet.

- Packet Type Description

| Packet Type | Response packet type | Sender | Related instructions |
| --- | --- | --- | --- |
| 0x01 | 0x81 | Client | Registration packet |
| 0x02 | 0x82 | Client | Heartbeat packet |
| 0x03 | 0x83 | Client | event packet |
| 0x04 | 0x84 | Client | Notification packet |
| 0x05 | 0x85 | Server | Read configuration parameters |
| 0x06 | 0x86 | Server | Set configuration parameters |
| 0x07 | 0x87 | Server | Status query |
| 0x08 | 0x88 | Server | Device control |
| 0x0A | 0x8A | Server | Read configuration parameter information (configuration parameter list supported by the device) |
| 0x0B | 0x8B | Server | Read status parameter information (list of status parameters supported by the device) |
| 0x0C | 0x8C | Server | Read control parameter information (list of control parameters supported by the device) |
| 0x0D | 0x8D | Server | Read event parameter information (event parameter list supported by the device) |
| 0x0E | 0x8D | Server | Upgrade start |
| 0x0F | 0x8F | Server | Upgrade data transfer |

- Interaction Modes

  - Interaction Description

    - Registration Phase

Client: After establishing a connection, actively send a registration packet to the Server.

If a registration response packet is required, the Client waits for it to return. When the registration response packet is received, the Client is registered successfully. If it is not received within the timeout period, the Client resends the registration packet. If the retransmission count is exceeded, registration is considered unsuccessful and the connection is disconnected and re-established.

If no registration response packet is required, registration is considered successful.

Server: after establishing a connection, wait to receive the Client registration packet.

If the Client registration packet is not received within the specified time, registration is considered unsuccessful and the connection is disconnected.

If a Client registration packet is received, parse it and check whether a link for the specified “Device ID” exists. If it does, disconnect the previous link and use the new link for communication.

When a registration response packet is required, send a registration response packet to the Client and consider the Client successfully registered.

When no registration response packet is required, consider the Client successfully registered.

If this is the Client’s first registration, the Server should query the Client’s parameter information for synchronization.

      - Registration Packet(0x01)

The data body format of the registration packet is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values recommended for TFLV are as follows:

| TAG | Description | Field Type | Length | Mandatory/Optional |
| --- | --- | --- | --- | --- |
| 0x8002 | Version | INT16U | 2 | O |
| 0x8003 | Packet Size | INT16U | 2 | O |
| 0x8004 | heartbeat interval | INT16U | 2 | O |
| 0x8005 | data encryption | INT8U | 1 | O |
| 0x8006 | Response timeout | INT16U | 2 | O |
| 0x8007 | Number of retransmissions after timeout | INT8U | 1 | O |
| 0x8008 | Parameter synchronization identifier | INT8U | 1 | O |
| 0x8009 | SIM card CCID | STRING | 20 | O |
| 0x800A | User binding relationship | INT8U | 1 | O |
| 0x800B | Online hold time | INT16U | 2 | O |
| 0x8081 | additional function identifier | INT32U | 4 | O |
| 0x8082 | Custom number | INT32U | 4 | O |

Registration Response Packet(0x81)

The data body format of the registration response packet is as follows:

None

If registration fails, set the corresponding status in the status identifier and return the packet to the requester.

    - Running Phase

After the Client and Server complete registration, they can exchange data, control information, and other information.

      - Heartbeat Packet(0x02)

When no data is exchanged between the Server and Client for a certain period, the Client should actively send a heartbeat packet to the Server to maintain the link between them.

Client:

If a heartbeat response packet is required, wait for it to return. Receipt of the response indicates successful transmission. If no response is received within the timeout period, resend the packet. If the retransmission count is exceeded, transmission is considered unsuccessful and the connection is disconnected and re-established.

If no heartbeat response packet is required, completion of transmission indicates successful transmission.

The data body format of the heartbeat packet is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

TFLV can carry status parameters and report status at the same time.

Heartbeat Response Packet(0x82)

When the Server receives a heartbeat packet, the link with the Client is considered normal. If a response is required, send a heartbeat response packet.

The heartbeat response packet format is as follows:

None

Event Packet(0x03)

When the Client needs to inform the Server of specific information, the Client should actively send an event packet to the Server.

Client:

If a response packet is required, wait for it to return. Receipt of the response indicates successful transmission. If no response is received within the timeout period, resend the packet. If the retransmission count is exceeded, transmission is considered unsuccessful and the connection is disconnected and re-established.

If no response packet is required, completion of transmission indicates successful transmission.

The data body format of the event packet is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Event Response Packet(0x83)

When the Server receives an event packet, it processes the related information. If a response is required, it sends a response packet.

The event response packet format is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Notification Packet(0x04)

When the Client needs to notify the Server of changes to configuration or status parameters, the Client should actively send a notification packet to the Server.

Client:

If a response packet is required, wait for it to return. Receipt of the response indicates successful transmission. If no response is received within the timeout period, resend the packet. If the retransmission count is exceeded, transmission is considered unsuccessful and the connection is disconnected and re-established.

If no response packet is required, completion of transmission indicates successful transmission.

The data body format of the notification packet is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Notification Response Packet(0x84)

When the Server receives a notification packet, it processes the related information. If a response is required, it sends a response packet.

The notification response packet format is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Read Configuration Parameter Request Packet(0x05)

When the Server needs to read configuration parameters from the Client, the Server should actively send a Read Configuration Parameter Request Packet to the Client.

In the TFLV structure, set Length and Type according to the corresponding values for the Tag, and fill Data Value with 0x00 for the corresponding length.

Server:

The read configuration parameter request packet requires waiting for a response. Receipt of the response indicates successful transmission. If no response is received within the timeout period, resend the request packet. If the retransmission count is exceeded, transmission is considered unsuccessful and the connection is disconnected and re-established.

The format of the Configuration parameter request packet is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Read Configuration Parameter Request Response Packet(0x85)

When the Client receives a Read Parameter Request Packet, it parses the packet and checks the CRC, length, and data type. After the checks and read operation are complete, it sets the corresponding status in the status field of the Flag in the TFLV structure. For status values, refer to “Appendix - Code Table - TFLV Result - Status Identifier”. The adjusted request-packet data body is returned as the response-packet data body.

The format of the Configuration Parameter Request Response Packet is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Set Configuration Parameter Request Packet(0x06)

When the Server needs to modify configuration parameters on the Client, the Server should actively send a Set Configuration Parameter Request Packet to the Client.

Server:

The Set Configuration Parameter Request Packet requires the Client to wait for a response packet. Receipt of the response packet indicates successful transmission. If no response is received within the timeout period, resend the Set Configuration Parameter Request Packet. If the retransmission count is exceeded, transmission is considered unsuccessful; disconnect and reconnect.

The format of the Configuration Parameter Request  Packet as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Set Configuration Parameter Request Response Packet(0x86)

When the Client receives a Set Configuration Parameter Request Packet, it parses the packet and checks the CRC, length, and data type. After the checks and setting operation are complete, it sets the corresponding status in the status field of the Flag in the TFLV structure. For status values, refer to “Appendix - Code Table - TFLV Result - Status Identifier”. The adjusted request-packet data body is returned as the response-packet data body.

The format of the Configuration Parameter Request Response Packet is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Status Query Request Packet(0x07)

When the Server needs to read status information from the Client, the Server should actively send a Status Query Request Packet to the Client.

In the TFLV structure, set Length and Type according to the corresponding values for the Tag, and fill Data Value with 0x00 for the corresponding length.

Server:

The status query request packet requires waiting for a response. Receipt of the response indicates successful transmission. If no response is received within the timeout period, resend the request packet. If the retransmission count is exceeded, transmission is considered unsuccessful and the connection is disconnected and re-established.

The format of the Status Query Request Packet is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Status Query Request Response Packet(0x87)

When the Client receives a Status Query Request Packet, it parses the packet and checks the CRC, length, and data type. After the checks and read operation are complete, it sets the corresponding status in the status field of the Flag in the TFLV structure. For status values, refer to “Appendix - Code Table - TFLV Result - Status Identifier”. The adjusted request-packet data body is returned as the response-packet data body.

The format of the Status Query Request Response Packet is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Device Control Request Packet(0x08)

When the Server needs to control the device, the Server should actively send a Device Control Request Packet to the Client.

In the TFLV structure, set Length and Type according to the corresponding values for the Tag, and fill Data Value with 0x00 for the corresponding length.

The format of the Device Control Request Packet is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Device Control Request Response Packet(0x88)

When the Receiver receives a Device Control Request Packet, it parses the packet and checks the CRC, length, and data type. After the checks and read operation are complete, it sets the corresponding status in the status field of the Flag in the TFLV structure. For status values, refer to “Appendix - Code Table - TFLV Result - Status Identifier”. The adjusted data body of the original request packet is returned as the response-packet data body.

For the operations performed by each control-command request packet, refer to the command TAG description.

The format of the Device Control Request Response Packet is as follows:

TFLV Structure

TFLV Structure

……

TFLV Structure

The TAG values that TFLV can carry are based on the “TAG Definition for Each Device Type”..

Read Configuration Parameter Information Request Packet(0x0A)

When the Server needs to read the Client’s configuration parameter information, including which configuration parameters the device supports, the Server should actively send a Read Configuration Parameter Information Request Packet to the Client.

Server:

The Read Configuration Parameter Information Request Packet requires waiting for a response packet. Receipt of the response packet indicates successful transmission. If no response is received within the timeout period, resend the Read Configuration Parameter Information Request Packet. If the retransmission count is exceeded, transmission is considered unsuccessful; disconnect and reconnect.

The format of the Read Configuration Parameter Information Request Packet is as follows:

None

Read Configuration Parameter Information Request Response Packet(0x8A)

When the Client receives a Read Configuration Parameter Information Request Packet, it returns the list of configuration-parameter TAGs supported by the device.

|  | Note: Because the number of Client parameters is not fixed, one request may require multiple response packets. |
| --- | --- |

The returned data body is formatted as follows:

Tag

Tag

……

Tag

TAG values are defined in “TAG Definition for Each Device Type”.

Read Status Parameter Information Request Packet(0x0B)

When the Server needs to read the Client’s status parameter information, including which status parameters the device supports, the Server should actively send a Read Status Parameter Information Request Packet to the Client.

Server:

The Read Status Parameter Information Request Packet requires waiting for a response packet. Receipt of the response packet indicates successful transmission. If no response is received within the timeout period, resend the Read Status Parameter Information Request Packet. If the retransmission count is exceeded, transmission is considered unsuccessful; disconnect and reconnect.

The format of the Read Status Parameter Information Request Packet is as follows:

None

Read Status Parameter Information Request Response Packet(0x8B)

When the Client receives a Read Status Parameter Information Request Packet, it returns the list of status-parameter TAGs supported by the device.

|  | Note: Because the number of Client parameters is not fixed, one request may require multiple response packets. |
| --- | --- |

The returned data body is formatted as follows:

Tag

Tag

……

Tag

TAG values are defined in “TAG Definition for Each Device Type”.

Read Control Parameter Information Request Packet(0x0C)

When the Server needs to read the Client’s control-command information, including which Control Parameters the device supports, the Server should actively send a Read Control Parameter Information Request Packet to the Client.

Server:

The Read Control Parameter Information Request Packet requires waiting for a response packet. Receipt of the response packet indicates successful transmission. If no response is received within the timeout period, resend the Read Control Parameter Information Request Packet. If the retransmission count is exceeded, transmission is considered unsuccessful; disconnect and reconnect.

The format of the Read Control Parameter Information Request Packet is as follows:

None

Read Control Parameter Information Request Response Packet(0x8C)

When the Client receives a Read Control Parameter Information Request Packet, it returns the list of control-command TAGs supported by the device.

|  | Note: Because the number of Client parameters is not fixed, one request may require multiple response packets. |
| --- | --- |

The returned data body is formatted as follows:

Tag

Tag

……

Tag

TAG values are defined in “TAG Definition for Each Device Type”.

Read Event Parameter Information Request Packet(0x0D)

When the Server needs to read the Client’s event parameter information, including which event parameters the device supports, the Server should actively send a Read Event Parameter Information Request Packet to the Client.

Server:

The Read Event Parameter Information Request Packet requires waiting for a response packet. Receipt of the response packet indicates successful transmission. If no response is received within the timeout period, resend the Read Event Parameter Information Request Packet. If the retransmission count is exceeded, transmission is considered unsuccessful; disconnect and reconnect.

The format of the Read Event Parameter Information Request Packet is as follows:

None

Read Event Parameter Information Request Response Packet(0x8D)

When the Client receives a Read Event Parameter Information Request Packet, it returns the list of event-parameter TAGs supported by the device.

|  | Note: Because the number of Client parameters is not fixed, one request may require multiple response packets. |
| --- | --- |

The returned data body is formatted as follows:

Tag

Tag

……

Tag

TAG values are defined in “TAG Definition for Each Device Type”.

      - Upgrade Startup Packet(0x0E)

The data body format of the upgrade startup packet is as follows. File size is in bytes, the file check code is the cumulative sum of the file data, and the timeout is in seconds:

File Size [4]

File Checksum [4]

Upgrade Timeout [2] [2]

Reserved [10]

Upgrade Startup Response Packet(0x8E)

The data body format of the upgrade startup response packet is as follows:

None

      - Upgrade Data Transfer Packet(0x0F)

The data body format of the upgrade data transfer packet is as follows. After all packets have been transferred, send a packet whose upgrade-packet offset equals the firmware size and whose upgrade-packet data length is 0 as the end marker:

Packet Offset [4]

Upgrade Packet Data [n]

Upgrade Data Transfer Response Packet(0x8F)

None

- 8. Appendix

  - Code Table

    - Status Identifier

| Value | Description |
| --- | --- |
| 0x00 | Normal packet ID |
| 0x01 | Wrong protocol version number |
| 0x03 | Abnormal subpacket sequence number |
| 0x04 | Wrong device type |
| 0x05 | Abnormal device number |
| 0x06 | Wrong packet type |
| 0x07 | Abnormal length |
| 0x08 | Upgrade failed |
| 0x7F | Other errors |

    - Function Identifier

| Value | Description |
| --- | --- |
| Bit0 | Reserve |
| Bit1 | 0: Must respond; 1: No need to respond |
| Bit2 | 0: Application data is not encrypted; 1: Application data is encrypted (AES128-CFB) |

    - Device Type

| Value | Description |
| --- | --- |
| 0x00 None | 0x00 None |
| 0x01 Smart Pill Box V1.2 | 0x01 Smart Pill Box V1.2 |
| 0x02 Smart Pill Box V2 | 0x02 Smart Pill Box V2 |

    - Device ID

| Value (binary) | Description |
| --- | --- |
| bit63~bit62: 00：MAC 01：IMEI 10: Reserved 11: Reserved | Device identification type. The MAC is filled in byte order, and the high bits are filled with 0s. The IMEI number is filled in according to the 8421-BCD code. |
| bit61 | Reserve. Filled with 0. |
| bit60 | Reserve. Filled with 0. |
| Bit59~bit0 | Device identification number |

    - TFLV Structure - Status Identification

| Value (binary) | Description |
| --- | --- |
| 000 | success |
| 001 | TAG is invalid |
| 010 | Invalid parameter type |
| 011 | The length of TFLV does not match the actual length of the value |
| 100 | Illegal value |
| 101 | operation failed |
| 111 | other |

    - TFLV Structure - Parameter Type

| Value (binary) | V String representation | Description |
| --- | --- | --- |
| 00000 | UNKONW | unknown |
| 00001 | INT8S | Signed 8-bit integer |
| 00010 | INT8U | unsigned 8-bit integer |
| 00011 | INT16S | Signed 16-bit integer |
| 00100 | INT16U | Unsigned 16-bit integer |
| 00101 | INT32S | Signed 32-bit integer |
| 00110 | INT32U | Unsigned 32-bit integer |
| 00111 | INT64S | Signed 64-bit integer |
| 01000 | INT64U | Unsigned 64-bit integer |
| 01001 | FLOAT | Single precision floating point number |
| 01010 | DOUBLE | Double precision floating point number |
| 01011 | STRING | string |
| 01100 | ARRAY | byte array |

  - CRC Algorithm C Code

typedef unsigned char INT8U;

typedef unsigned short int INT16U;

/**************************************************************************

* Function name: CalcCRC

* Function description: single-byte CRC

* Tables accessed:

* Tables modified:

* Input parameters:

INT8U ucData : Character

INT16U usCRCIn : Previous CRC value

* Output parameters:

* Return value:

Returns the CRC value

* OtherDescription:

* Revision date, version, author, and revision description

* -----------------------------------------------

**************************************************************************/

static INT16U CalcCRC(INT8U ucData, INT16U usCRCIn)

{

INT16U usIndex = 0;

INT16U usCheck = 0;

usCRCIn =usCRCIn ^ ucData;

for (usIndex = 0; usIndex< 8; ++usIndex)

{

usCheck = usCRCIn& 1;

usCRCIn = usCRCIn>> 1;

usCRCIn = usCRCIn& 0x7FFF;

if (1 == usCheck)

{

usCRCIn = usCRCIn ^ 0xA001;

}

usCRCIn = usCRCIn& 0xFFFF;

}

return usCRCIn;

}

/**************************************************************************

* Function name: CRC16

* Function description: CRC16

* Tables accessed:

* Tables modified:

* Input parameters:

UCHAR *pucDst : Destination buffer address

INT16U usDstLen : Maximum destination buffer length

* Output parameters:

* Return value:

Returns the length of the processed data

* OtherDescription:

* Revision date, version, author, and revision description

* -----------------------------------------------

**************************************************************************/

INT16U CRC16(INT8U *pucSrc, INT16U usSrcLen)

{

INT16U usCRC = 0;

INT16U usIndex = 0;

INT8U *pucTemp = NULL;

usCRC = 0xFFFF;

pucTemp = pucSrc;

for (usIndex = 0; usIndex<usSrcLen; ++usIndex)

{

usCRC = CalcCRC(*pucTemp, usCRC);

++pucTemp;

}

return usCRC;

}

  - TAG Definition - Device Type 01

| TAG | Description | Type | Length | Maximum | Minimum | Value Range |
| --- | --- | --- | --- | --- | --- | --- |
| Configuration Parameters |  |  |  |  |  |  |
| 0x1001 | Language | INT8U | - | 1 | 0 | 0: Default 1: English |
| 0x1002 | Date format | INT8U | - | 2 | 0 | 0: Year/Month/Day 1: Day/Month/Year 2: Month/Day/Year |
| 0x1003 | Time format | INT8U | - | 1 | 0 | 0: 24-hour clock 1: 12-hour clock |
| 0x1004 | Alarm start year | INT16U | - | 2100 | 2000 |  |
| 0x1005 | Alarm start month | INT8U | - | 12 | 1 |  |
| 0x1006 | Alarm start day | INT8U | - | 31 | 1 |  |
| 0x1007 | Alarm end year | INT16U | - | 2100 | 2000 |  |
| 0x1008 | Alarm end month | INT8U | - | 12 | 1 |  |
| 0x1009 | Alarm end day | INT8U | - | 31 | 1 |  |
| 0x1011 | Alarm duration | INT16U | - | 3600 | 0 | (Reserved for future use) Unit: minutes 0 means continuous ringing |
| 0x1012 | Alarm ringtone type | INT8U | - | 4 | 0 | 0: None 1: Ringtone 1 2: Ringtone 2 3: Ringtone 3 4: Ringtone 4 |
| 0x1013 | Alarm ringtone volume | INT8U | - | 2 | 0 | 0: High 1: Medium 2: Low 3: Mute |
| 0x1014 | Automatic Time Calibration | INT8U | - | 1 | 0 | 0: Disabled 1: Enabled |
| 0x1015 | Time Zone | INT16S | - | +1400 | -1200 |  |
| 0x1016 | Device ID | INT64U | 8 | - | - | Reserved for future use |
| 0x1017 | Medication Retrieval Timeout | INT32U | - | 86400 | 0 | 0 means that the device default value is used |
| 0x1018 | Missed Medication Retrieval Time | INT32U | - | 86400 | 0 | 0 means that the device default value is used |
| 0X1019 | Early Medication Retrieval Validity Period | INT32U | - | 3600 | 0 | Reserved for future use 0 means that early medication retrieval is disabled |
| 0x101A | Current Medication Compartment Number | INT8U | - | 28 | 0 | Reserved for future use |
| 0x101B | Total Number of Medication Compartments | INT8U | - | 29 | 29 | Reserved for future use |
| 0x101C | Loaded Medication Compartments | INT8U | - | 28 | 0 |  |
| 0x101D | Remaining Medication Compartments | INT8U | - | 28 | 0 | Reserved for future use |
| 0x1021 | Alarm1Hour | INT8U | - | 23 | 0 |  |
| 0x1022 | Alarm2Hour | INT8U | - | 23 | 0 |  |
| 0x1023 | Alarm3Hour | INT8U | - | 23 | 0 |  |
| 0x1024 | Alarm4Hour | INT8U | - | 23 | 0 |  |
| 0x1025 | Alarm5Hour | INT8U | - | 23 | 0 |  |
| 0x1026 | Alarm6Hour | INT8U | - | 23 | 0 |  |
| 0x1027 | Alarm7Hour | INT8U | - | 23 | 0 |  |
| 0x1028 | Alarm8Hour | INT8U | - | 23 | 0 |  |
| 0x1029 | Alarm9Hour | INT8U | - | 23 | 0 |  |
| 0x1031 | Alarm1Minute | INT8U | - | 59 | 0 |  |
| 0x1032 | Alarm2Minute | INT8U | - | 59 | 0 |  |
| 0x1033 | Alarm3Minute | INT8U | - | 59 | 0 |  |
| 0x1034 | Alarm4Minute | INT8U | - | 59 | 0 |  |
| 0x1035 | Alarm5Minute | INT8U | - | 59 | 0 |  |
| 0x1036 | Alarm6Minute | INT8U | - | 59 | 0 |  |
| 0x1037 | Alarm7Minute | INT8U | - | 59 | 0 |  |
| 0x1038 | Alarm8Minute | INT8U | - | 59 | 0 |  |
| 0x1039 | Alarm9Minute | INT8U | - | 59 | 0 |  |
| 0x1051 | Do Not Disturb Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1052 | Do Not Disturb start hour | INT8U | - | 23 | 0 |  |
| 0x1053 | Do Not Disturb start minute | INT8U | - | 59 | 0 |  |
| 0x1054 | Do Not Disturb end hour | INT8U | - | 23 | 0 |  |
| 0x1055 | Do Not Disturb end minute | INT8U | - | 59 | 0 |  |
| 0x1061 | Emergency Call Trigger Time | INT16U | - | 3600 | 0 | Unit: seconds; an emergency call is triggered after inversion continues for N seconds; 0 disables this function. |
| 0x1062 | Key Tone Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1063 | Alarm Ring Pause Duration | INT16U | - | 1440 | 0 | Unit: minutes; ringing resumes after N minutes of pause; 0 mutes the current alarm. |

| TAG | Description | Type | Length | Maximum | Minimum | Value Range |
| --- | --- | --- | --- | --- | --- | --- |
| General Status Parameters |  |  |  |  |  |  |
| 0x8001 | Device Type | INT8U | - | 255 | 1 | Deprecated 0: Unknown 1: Smart Pill Box V1.2 |
| 0x8002 | Version | INT16U | - | 65535 | 0 |  |
| 0x8003 | Packet Size | INT16U | - | 1400 | 0 | Unit: bytes |
| 0x8004 | Heartbeat packet interval | INT16U | - | 65535 | 0 | Unit: seconds |
| 0x8005 | Data Encryption | INT8U | - | 1 | 0 | 0: No 1: Yes |
| 0x8006 | Response Timeout | INT16U | - | 120 | 10 | Unit: seconds |
| 0x8007 | Retransmission Count After Timeout | INT8U | - | 10 | 0 |  |
| 0x8008 | Parameter Synchronization Identifier | INT8U | - | 1 | 0 | 0: Non-forced synchronization 1: Forced synchronization |
| 0x8009 | SIM Card CCID | STRING | 20 | - | - |  |
| 0x800A | User Binding Relationship | INT8U | - | 1 | 0 | 0: Not forced 1: Forced |
| 0x800B | Online Hold Time | INT16U | - | 65535 | 0 | Unit: seconds; enters sleep mode after the online hold time; 0 means no sleep |
| 0x8081 | Additional Function Identifier | INT32U | - | 0xFFFFFFFF | 0 | Bit0: Server switching Bit1: Bluetooth OTA |
| 0x8082 | Custom Number | INT32U | - | 0xFFFFFFFF | 0 | Refer to the Custom Number description file for the value definition |
| Status Parameters |  |  |  |  |  |  |
| 0x8101 | Medication Status | INT8U | - | 2 | 0 | 0: Normal 1: Medication low 2: No medication |
| 0x8103 | Battery Level | INT8U | - | 100 | 1 | Battery percentage; 1 means 1%, 100 means 100%; not displayed while charging? |
| 0x8104 | Battery Status | INT8U | - | 4 | 0 | 0: Normal 1: Fully charged 2: Low battery 3: Charging 4: No battery |
| 0x8105 | Pill Tray ID | STRING | 20 | - | - | Deprecated |
| 0x8106 | Low battery voltage | INT8U | - | 1 | 0 | Deprecated 0: Normal battery voltage 1: Low battery voltage |
| 0x8107 | Lid Status | INT8U | - | 1 | 0 | 0: Lid closed 1: Lid open |
| 0x8108 | Pill Tray Rotation Fault | INT8U | - | 1 | 0 | 0: Normal 1: Fault |
| 0x8109 | DC Power Status | INT8U | - | 1 | 0 | 0: DC power disconnected 1: DC power supplied |
| 0x810A | Wi-Fi signal strength | INT16S | - | 300 | -300 |  |
| 0x810B | GSM signal strength | INT16S | - | 300 | -300 |  |
| 0x810C | Wi-Fi signal level | INT8U | - | 4 | 0 |  |
| 0x810D | GSM signal level | INT8U | - | 4 | 0 |  |
| 0x811A | Current Medication Compartment Number | INT8U | - | 28 | 0 |  |
| 0x811B | Total Number of Medication Compartments | INT8U | - | 29 | 29 |  |
| 0x811C | Loaded Medication Compartments | INT8U | - | 28 | 0 | Reserved for future use |
| 0x811D | Remaining Medication Compartments | INT8U | - | 28 | 0 |  |
| 0x8122 | Pill Tray Reset Fault | INT8U | - | 1 | 0 | 0: Normal 1: Fault |
| 0x8123 | Emergency Call Status | INT8U | - | 1 | 0 | 0: Normal 1: Emergency state |
| 0x8131 | Alarm 1 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 4: Timed out while waiting for dispensing 6: Missed medication 7: Medication taken |
| 0x8132 | Alarm 2 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 4: Timed out while waiting for dispensing 6: Missed medication 7: Medication taken |
| 0x8133 | Alarm 3 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 4: Timed out while waiting for dispensing 6: Missed medication 7: Medication taken |
| 0x8134 | Alarm 4 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 4: Timed out while waiting for dispensing 6: Missed medication 7: Medication taken |
| 0x8135 | Alarm 5 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 4: Timed out while waiting for dispensing 6: Missed medication 7: Medication taken |
| 0x8136 | Alarm 6 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 4: Timed out while waiting for dispensing 6: Missed medication 7: Medication taken |
| 0x8137 | Alarm 7 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 4: Timed out while waiting for dispensing 6: Missed medication 7: Medication taken |
| 0x8138 | Alarm 8 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 4: Timed out while waiting for dispensing 6: Missed medication 7: Medication taken |
| 0x8139 | Alarm 9 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 4: Timed out while waiting for dispensing 6: Missed medication 7: Medication taken |
| 0x8140 | Pill Tray Rotation Status | INT8U | - | 1 | 0 | 0: None 1: Rotating |
| 0x8141 | Medication Pause Status | INT8U | - | 1 | 0 | 0: Normal 1: Paused |

| TAG | Description | Type | Length | Maximum | Minimum | Value Range |
| --- | --- | --- | --- | --- | --- | --- |
| General Control Parameters |  |  |  |  |  |  |
| 0xA001 | Device Restart | INT8U | - | 1 | 0 | 0: Immediate restart 1: Delayed restart |
| 0xA002 | Restore Factory Settings | INT8U | - | 0 | 0 |  |
| 0xA003 | Cancel forced synchronization | INT8U | - | 0 | 0 |  |
| 0xA004 | Device Re-registration | INT8U | - | 0 | 0 |  |
| 0xA011 | Set Heartbeat Interval | INT16U | - | 65535 | 0 | Unit: seconds |
| 0xA012 | Set Online Hold Time | INT16U | - | 65535 | 0 | Reserved for future use Unit: seconds; enters sleep mode after the online hold time; 0 means no sleep |
| 0xA021 | Set Server IP | STRING | 16 | - | - |  |
| 0xA022 | Set Server Domain Name | STRING | 64 | - | - |  |
| 0xA023 | Set Server Port | INT16U | - | 65535 | 0 |  |
| Control Parameters |  |  |  |  |  |  |
| 0xA101 | Device Time Calibration | STRING | 20 | - | - | 2001-01-02T20:05:04 |
| 0xA102 | Mute Device | INT8U | - | 1 | 1 | Mute Current Alarm |
| 0xA103 | Pill Tray Reset | INT8U | - | 1 | 1 |  |
| 0xA121 | Set Wi-Fi SSID | STRING | 32 | - | - | Reserved for future use |
| 0xA122 | Set Wi-Fi password | STRING | 32 | - | - | Reserved for future use |
| 0xA123 | Early Medication Retrieval | INT8U | - | 0 | 0 |  |
| 0xA124 | Rotate to Specified Medication Compartment | INT8U | - | 28 | 0 | 0 means no rotation; a value greater than 0 rotates to the corresponding medication compartment |
| 0xA125 | Medication Pause Duration Setting | INT8U | - | 255 | 0 | 0 means normal; a value greater than 0 prohibits medication for the specified number of minutes |

| TAG | Description | Type | Length | Maximum | Minimum | Value Range |
| --- | --- | --- | --- | --- | --- | --- |
| General Event Parameters |  |  |  |  |  |  |
| 0xC001 | Exit Request | INT8U | - | 1 | 0 | Reserved for future use 0: Device-initiated exit 1: Sleep-mode exit |
| Event Parameters |  |  |  |  |  |  |
| 0xC101 | Time Calibration Request | INT16S | - | 1400 | -1200 |  |
| 0xC201 | Medication Retrieval Record | INT8U | - | 5 | 0 | 0: Waiting for medication retrieval 1: On-time retrieval successful 2: Retrieval wait timed out 3: Retrieval after timeout successful 4: Medication not retrieved 5: Early retrieval successful |
| 0xC202 | Alarm Time | STRING | 20 | - | - | 2001-01-02T20:05:04 |
| 0xC203 | Pill Tray ID | STRING | 20 | - | - | Reserved for future use |
| 0xC204 | Current Medication Compartment Number | INT8U | - | 28 | 0 |  |
| 0xC205 | Pill Tray Reset Report | INT8U | - | 2 | 0 | Deprecated 0: Pill tray reset started 1: Pill tray reset successful 2: Pill tray reset failed |
| 0xC206 | Device Startup Report | INT8U | - | 0 | 0 | Deprecated |
| 0xC207 | Medication Retrieval Trigger Identifier | INT8U | - | 2 | 0 | 0: On-time medication retrieval 1: Early medication retrieval 2: Delayed medication retrieval |
| 0xC208 | Medication Retrieval Time | STRING | 20 | - | - | 2001-01-02T20:05:04 |
| 0xC301 | Pill Tray Reset Report | INT8U | - | 2 | 0 | Reserved for future use 0: Pill tray reset started 1: Pill tray reset successful 2: Pill tray reset failed |
| 0xC401 | Network Configuration Result Report | INT8U | - | 1 | 0 | Reserved for future use 0: Success 1: Failure |
| 0xC402 | Network Configuration Identifier | INT16U | - | 65535 | 0 | Reserved for future use |
| 0xC403 | Network Configuration Method | INT8U | - | 1 | 0 | Reserved for future use 0: Quick network configuration 1: Manual network configuration |
| 0xC501 | Factory Reset Report | INT8U | - | 1 | 0 | Reserved for future use 0: Local operation 1: Remote operation |
| 0xC601 | Device Startup Report | INT8U | - | 0 | 0 | Reserved for future use |

  - TAG Definition - Device Type 02

| TAG | Description | Type | Length | Maximum | Minimum | Value Range |
| --- | --- | --- | --- | --- | --- | --- |
| Configuration Parameters |  |  |  |  |  |  |
| 0x1001 | Language | INT8U | - | 1 | 0 | 0: Default 1: English |
| 0x1002 | Date format | INT8U | - | 2 | 0 | 0: Year/Month/Day 1: Day/Month/Year 2: Month/Day/Year |
| 0x1003 | Time format | INT8U | - | 1 | 0 | 0: 24-hour clock 1: 12-hour clock |
| 0x1004 | Alarm start year | INT16U | - | 2100 | 2000 |  |
| 0x1005 | Alarm start month | INT8U | - | 12 | 1 |  |
| 0x1006 | Alarm start day | INT8U | - | 31 | 1 |  |
| 0x1007 | Alarm end year | INT16U | - | 2100 | 2000 |  |
| 0x1008 | Alarm end month | INT8U | - | 12 | 1 |  |
| 0x1009 | Alarm end day | INT8U | - | 31 | 1 |  |
| 0x100A | Alarm Valid-Date Switch | INT8U | - | 1 | 0 | 0: Off (the alarm is always valid) 1: On (the alarm is valid only within the valid dates) |
| 0x100B | Key Tone Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x100C | Safety Mode (Child Lock/Key Lock) Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x100D | Early Medication Retrieval Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x100E | Emergency Call Switch | INT8U | - | 3 | 0 | 0: Off 1: On |
| 0x1012 | Alarm Ringtone Type | INT8U | - | 3 | 0 | 0: None 1: Ringtone 1 2: Ringtone 2 3: Ringtone 3 4: Ringtone 4 |
| 0x1013 | Volume Level | INT8U | - | 3 | 0 | 0: High 1: Medium 2: Low 3: Mute |
| 0x1014 | Automatic Time Calibration Switch | INT8U | - | 1 | 0 | 0: Disabled 1: Enabled |
| 0x1015 | Time Zone | INT16S | - | +1400 | -1200 |  |
| 0x1017 | Medication Retrieval Warning Time | INT32U | - | 86400 | 0 | 0 means that the device default value is used |
| 0x1018 | Missed Medication Retrieval Time | INT32U | - | 86400 | 0 | 0 means that the device default value is used |
| 0x1019 | Missed-medication dispensing switch | INT8U | - | 1 | 0 | Whether dispensing is allowed after medication retrieval is missed 0: Disabled (not allowed) 1: Enabled (allowed) |
| 0x101A | Current Medication Compartment Number | INT8U | - | 28 | 0 | Reserved for future use |
| 0x101C | Loaded Medication Compartments | INT8U | - | 28 | 0 |  |
| 0x101D | Low-Medication Warning Compartment Count | INT8U | - | 28 | 0 | 0 means no warning |
| 0x1021 | Alarm1Hour | INT8U | - | 23 | 0 |  |
| 0x1022 | Alarm2Hour | INT8U | - | 23 | 0 |  |
| 0x1023 | Alarm3Hour | INT8U | - | 23 | 0 |  |
| 0x1024 | Alarm4Hour | INT8U | - | 23 | 0 |  |
| 0x1025 | Alarm5Hour | INT8U | - | 23 | 0 |  |
| 0x1026 | Alarm6Hour | INT8U | - | 23 | 0 |  |
| 0x1027 | Alarm7Hour | INT8U | - | 23 | 0 |  |
| 0x1028 | Alarm8Hour | INT8U | - | 23 | 0 |  |
| 0x1029 | Alarm9Hour | INT8U | - | 23 | 0 |  |
| 0x1031 | Alarm1Minute | INT8U | - | 59 | 0 |  |
| 0x1032 | Alarm2Minute | INT8U | - | 59 | 0 |  |
| 0x1033 | Alarm3Minute | INT8U | - | 59 | 0 |  |
| 0x1034 | Alarm4Minute | INT8U | - | 59 | 0 |  |
| 0x1035 | Alarm5Minute | INT8U | - | 59 | 0 |  |
| 0x1036 | Alarm6Minute | INT8U | - | 59 | 0 |  |
| 0x1037 | Alarm7Minute | INT8U | - | 59 | 0 |  |
| 0x1038 | Alarm8Minute | INT8U | - | 59 | 0 |  |
| 0x1039 | Alarm9Minute | INT8U | - | 59 | 0 |  |
| 0x1041 | Alarm1 Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1042 | Alarm2 Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1043 | Alarm3 Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1044 | Alarm4 Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1045 | Alarm5 Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1046 | Alarm6 Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1047 | Alarm7 Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1048 | Alarm8 Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1049 | Alarm9 Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1051 | Do Not Disturb Switch | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x1052 | Do Not Disturb start hour | INT8U | - | 23 | 0 |  |
| 0x1053 | Do Not Disturb start minute | INT8U | - | 59 | 0 |  |
| 0x1054 | Do Not Disturb end hour | INT8U | - | 23 | 0 |  |
| 0x1055 | Do Not Disturb end minute | INT8U | - | 59 | 0 |  |
| 0x1063 | Alarm Ring Pause Duration | INT16U | - | 1440 | 0 | Unit: minutes; ringing resumes after N minutes of pause; 0 mutes the current alarm. |

| TAG | Description | Type | Length | Maximum | Minimum | Value Range |
| --- | --- | --- | --- | --- | --- | --- |
| General Status Parameters |  |  |  |  |  |  |
| 0x8002 | Version | INT16U | - | 65535 | 0 |  |
| 0x8003 | Packet Size | INT16U | - | 1400 | 0 | Unit: bytes |
| 0x8004 | Heartbeat packet interval | INT16U | - | 65535 | 0 | Unit: seconds |
| 0x8005 | Data Encryption | INT8U | - | 1 | 0 | 0: No 1: Yes |
| 0x8006 | Response Timeout | INT16U | - | 120 | 10 | Unit: seconds |
| 0x8007 | Retransmission Count After Timeout | INT8U | - | 10 | 0 |  |
| 0x8008 | Parameter Synchronization Identifier | INT8U | - | 1 | 0 | 0: Non-forced synchronization 1: Forced synchronization |
| 0x8009 | SIM Card CCID | STRING | 20 | - | - |  |
| 0x800A | User Binding Relationship | INT8U | - | 1 | 0 | 0: Not forced 1: Forced |
| 0x800B | Online Hold Time | INT16U | - | 65535 | 0 | Unit: seconds; enters sleep mode after the online hold time; 0 means no sleep |
| 0x8081 | Additional Function Identifier | INT32U | - | 0xFFFFFFFF | 0 | Bit0: Server switching Bit1: Bluetooth OTA Bit2: Remote upgrade |
| 0x8082 | Custom Number | INT32U | - | 0xFFFFFFFF | 0 | Refer to the Custom Number description file for the value definition |
| Status Parameters |  |  |  |  |  |  |
| 0x8101 | Medication Status | INT8U | - | 2 | 0 | 0: Medication normal 1: Medication low 2: No medication |
| 0x8102 | Child Lock (Key Lock) Status | INT8U | - | 1 | 0 | 0: Unlocked 1: Locked |
| 0x8103 | Battery Level | INT8U | - | 100 | 1 | Battery percentage; 1 means 1%, 100 means 100%; not displayed while charging? |
| 0x8104 | Battery Status | INT8U | - | 4 | 0 | 0: Normal 1: Fully charged 2: Low battery 3: Charging 4: No battery |
| 0x8105 | Do Not Disturb Status | INT8U | - | 2 | 0 | 0: Off 1: On 2: Enabled and active |
| 0x8106 | Medication Cup Status | INT8U | - | 1 | 0 | 0: Medication cup removed 1: Medication cup inserted |
| 0x8107 | Pill Tray Lock Status | INT8U | - | 1 | 0 | 0: Unlocked 1: Locked |
| 0x8109 | DC Power Status | INT8U | - | 1 | 0 | 0: Off 1: On |
| 0x810A | Wi-Fi signal strength | INT16S | - | 300 | -300 |  |
| 0x810B | GSM signal strength | INT16S | - | 300 | -300 |  |
| 0x810C | Wi-Fi signal level | INT8U | - | 3 | 0 |  |
| 0x810D | GSM signal level | INT8U | - | 3 | 0 |  |
| 0x810E | Temperature | INT8S | - | -40 | 120 | Unit: °C |
| 0x810F | Humidity | INT8U | - | 100 | 0 | Unit: %RH |
| 0x8111 | Temperature and Humidity Status | INT8U | - | 1 | 0 | 0: Normal 1: Alarm |
| 0x8112 | Emergency Call Status | INT8U | - | 1 | 0 | 0: Normal 1: Emergency call in progress |
| 0x811A | Current Medication Compartment Number | INT8U | - | 28 | 0 |  |
| 0x811B | Total Number of Medication Compartments | INT8U | - | 29 | 29 |  |
| 0x811D | Remaining Medication Compartments | INT8U | - | 28 | 0 |  |
| 0x8121 | Pill Tray Rotation Fault Status | INT8U | - | 1 | 0 | 0: Normal 1: Fault |
| 0x8122 | Pill Tray Reset Fault Status | INT8U | - | 1 | 0 | 0: Normal 1: Fault |
| 0x8123 | Medication Compartment Pusher Fault Status | INT8U | - | 1 | 0 | 0: Normal 1: Fault |
| 0x8124 | Medication Compartment Door Fault Status | INT8U | - | 1 | 0 | 0: Normal 1: Fault |
| 0x8125 | Key Fault Status | INT8U | - | 1 | 0 | 0: Normal 1: Fault |
| 0x8131 | Alarm 1 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 3: Waiting for medication retrieval 4: Timed out while waiting for dispensing 5: Timed out while waiting for medication retrieval 6: Missed medication 7: Medication taken |
| 0x8132 | Alarm 2 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 3: Waiting for medication retrieval 4: Timed out while waiting for dispensing 5: Timed out while waiting for medication retrieval 6: Missed medication 7: Medication taken |
| 0x8133 | Alarm 3 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 3: Waiting for medication retrieval 4: Timed out while waiting for dispensing 5: Timed out while waiting for medication retrieval 6: Missed medication 7: Medication taken |
| 0x8134 | Alarm 4 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 3: Waiting for medication retrieval 4: Timed out while waiting for dispensing 5: Timed out while waiting for medication retrieval 6: Missed medication 7: Medication taken |
| 0x8135 | Alarm 5 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 3: Waiting for medication retrieval 4: Timed out while waiting for dispensing 5: Timed out while waiting for medication retrieval 6: Missed medication 7: Medication taken |
| 0x8136 | Alarm 6 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 3: Waiting for medication retrieval 4: Timed out while waiting for dispensing 5: Timed out while waiting for medication retrieval 6: Missed medication 7: Medication taken |
| 0x8137 | Alarm 7 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 3: Waiting for medication retrieval 4: Timed out while waiting for dispensing 5: Timed out while waiting for medication retrieval 6: Missed medication 7: Medication taken |
| 0x8138 | Alarm 8 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 3: Waiting for medication retrieval 4: Timed out while waiting for dispensing 5: Timed out while waiting for medication retrieval 6: Missed medication 7: Medication taken |
| 0x8139 | Alarm 9 status | INT8U | - | 7 | 0 | 0: None 1: Preparing to dispense medication 2: Waiting for dispensing 3: Waiting for medication retrieval 4: Timed out while waiting for dispensing 5: Timed out while waiting for medication retrieval 6: Missed medication 7: Medication taken |

| TAG | Description | Type | Length | Maximum | Minimum | Value Range |
| --- | --- | --- | --- | --- | --- | --- |
| General Control Parameters |  |  |  |  |  |  |
| 0xA001 | Device Restart | INT8U | - | 1 | 0 | 0: Immediate restart 1: Delayed restart |
| 0xA002 | Restore Factory Settings | INT8U | - | 0 | 0 |  |
| 0xA003 | Cancel forced synchronization | INT8U | - | 0 | 0 |  |
| 0xA004 | Device Re-registration | INT8U | - | 0 | 0 |  |
| 0xA011 | Set Heartbeat Interval | INT16U | - | 65535 | 0 | Unit: seconds |
| 0xA012 | Set Online Hold Time | INT16U | - | 65535 | 0 | Reserved for future use Unit: seconds; enters sleep mode after the online hold time; 0 means no sleep |
| 0xA021 | Set Server IP | STRING | 16 | - | - |  |
| 0xA022 | Set Server Domain Name | STRING | 64 | - | - |  |
| 0xA023 | Set Server Port | INT16U | - | 65535 | 0 |  |
| Control Parameters |  |  |  |  |  |  |
| 0xA101 | Device Time Calibration | STRING | 20 | - | - | 2001-01-02T20:05:04 |
| 0xA102 | Mute Device | INT8U | - | 1 | 1 | Mute Current Alarm |
| 0xA103 | Pill Tray Reset | INT8U | - | 1 | 1 |  |
| 0xA123 | Early Medication Retrieval | INT8U | - | 0 | 0 |  |

| TAG | Description | Type | Length | Maximum | Minimum | Value Range |
| --- | --- | --- | --- | --- | --- | --- |
| General Event Parameters |  |  |  |  |  |  |
| 0xC001 | Exit Request | INT8U | - | 1 | 0 | Reserved for future use 0: Device-initiated exit 1: Sleep-mode exit |
| Event Parameters |  |  |  |  |  |  |
| 0xC201 | Medication Alarm ID | INT8U | - | 8 | 0 | 0: Alarm1 1: Alarm2 2: Alarm3 3: Alarm4 4: Alarm5 5: Alarm6 6: Alarm7 7: Alarm8 8: Alarm9 |
| 0xC202 | Medication Alarm Time | STRING | 20 | - | - | 2001-01-02T20:05:04 |
| 0xC203 | Current Medication Time | STRING | 20 | - | - | 2001-01-02T20:05:04 |
| 0xC204 | Medication Compartment Number | INT8U | - | 28 | 0 |  |
| 0xC205 | Medication Retrieval Method | INT8U | - | 2 | 0 | 0: On-time medication retrieval 1: Early medication retrieval 2: Delayed medication retrieval |
| 0xC206 | Medication Result | INT8U | - | 3 | 0 | 0: Medication taken on time 1: Medication taken late (after warning) 2: Abnormal medication (taken after a missed-dose event) 3: Medication missed |